UNIVERSITY OF ASIA PACIFIC

Department of Computer Science & Engineering

SOHO

Course Code	 :	CSE 320

Course Title	 :	Computer Networks Lab

Submitted by:                                                                 Submitted To: Name	:	Sheikh Nafez Sadnan              Md. Akhtaruzzamman Adnan Reg. No.  :		20101106                                             Assistant Professor

Roll No.     :	106                                                          Department of CSE

Section      :	B (2)                                                    University of Asia Pacific

Project Small Office Home Office (SOHO) Case Study and Requirements:
XYZ company is a fast-growing company in Eastern Australia with more than 2 million customers globally. The company deals with selling and buying of food items, which are basically operated from the headquarters. The company is intending to open a branch near the local village Bonalbo. Thus, the company requires young IT graduates to design the network for the branch. The network is intended to operate separately from the HQ network. Being a small network, the company has the following requirements during implementation;

- One router and one switch to be used (all CISCO products).

- 3 departments (Admin/IT, Finance/HR and Customer service/Reception).

- Each department is required to be in different VIANS.

- Each department is required to have a wireless network for the users.

- Host devices in the network are required to obtain IPv4 address automatically.

- Devices in all the departments are required to communicate with each other.

Assume the ISP gave out a base network of 192.168.1.0, you as the young network engineer who has been hired, design and implement a network considering the above requirements.

Technologies Implemented:

- Creating a Simple Network using a Router and Access Layer Switch.

- Connecting Networking devices with Correct cabling.

- Creating VLANs and assigning ports VLAN numbers.

- Subnetting and IP Addressing.

- Configuring Inter-VLAN Routing (Router on a stick).

- Configuring DHCP Server (Router as the DHCP Server).

- Configuring WLAN or wireless network (Cisco Access Point).

- Host Device Configurations.

- Test and Verifying Network Communication.

How to implement SOHO

Step-1: Open a project in Cisco Packet Tracer. Take a router, printer, access point & PC from the drop-down menu. The amount of these equipment’s depends on the architecture of the project.

Step-2: Rename the equipment’s to avoid confusion.

Step-3: Choose wires and give connections.

Step-4: Give section names and add colors.

Step-5: Give network ID subnet mask.

For admin/IT: 192.168.1.0/26

For Finance/HR: 192.168.1.64/26

For CS/Reception: 192.168.1.128/26

Step-6: Configure the VLAN. Go to switch then terminal and write down-

enable

configure terminal

int range fa0/2-4

switchport mode access

switchport access vlan 10

Similar process for other ports:

enable

configure terminal

int range fa0/5-7

switchport mode access

switchport access vlan 20

enable

configure terminal

int range fa0/8-10

switchport mode access

switchport access vlan 30

then,

do write

exit

do show start

Interfaces will show-

Step-7: Implement the access points-

Step-8: Configure the router. Go to terminal and write-

int fa 0/1

switchport mode trunk

do write

Step-9: Configure the remaining router configuration. Go to terminal again and write-

enable

configure terminal

int gig0/0

no shut

do write

Step-10: Configure inter VLAN routing and DHCP server. Starting with creating sub interface, Go to router terminal and write-

int gig0/0.10

encapsulation dot1Q 10

ip address 192.168.1.1 255.255.255.192

exit

int gig0/0.10

encapsulation dot1Q 20

ip address 192.168.1.65 255.255.255.192

exit

int gig0/0.30

encapsulation dot1Q 30

ip address 192.168.1.129 255.255.255.192

exit

do show start

Step-11: Configure DHCP server. Starting with creating sub interface, Go to router terminal and write-

service DHCP

dhcp pool Admin-pool

network 192.168.1.0 255.255.255.192

default-router 192.168.1.1

dns-server 192.168.1.1

domain-name Admin.com

exit

Similarly create another pool-

service DHCP

dhcp pool Finance-pool

network 192.168.1.64 255.255.255.192

default-router 192.168.1.65

dns-server 192.168.1.65

domain-name Finance.com

exit

Similarly create another pool-

service DHCP

dhcp pool CS-pool

network 192.168.1.128 255.255.255.192

default-router 192.168.1.129

dns-server 192.168.1.129

domain-name CS.com

exit

do write

Step-12: Test communication. Select DHCP for PC and access points.

Step-13: Pick a smartphone and configure it-

DHCP request is successful-

Now the smartphone is connected-

Similarly, take a laptop and configure it-

Turn off the laptop, remove existing and add WPC300N the turn on it-

Laptop is connected-

Similarly, connect a tab-

Tab is connected-

Step-14: Test communication. It should succeed-

Overall screenshot of the whole architecture-

Detailed view-

---------------------------THANK YOU FOR READING--------------------------